Experiment 1

AIM: Flashing of LED using AT89C51 Microcontroller using Proteus

Objective:

- To interface an LED with the AT89C51 microcontroller in the Proteus simulation environment.

- To write and execute a program for blinking the LED at regular time intervals.

- To understand the use of digital output ports and delay generation in the AT89C51.

Program:
ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END
OUTPUT:

Experiment 2

AIM: Generation Of Square Wave Using Proteus

Objective:

- To generate a square wave using one of the output pins of the AT89C51 microcontroller.

- To simulate the square wave generation using Proteus and observe the output waveform.

- To understand timer/delay concepts used for waveform generation in embedded systems.

Program:
ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END
OUTPUT:

Experiment 3

AIM: Stepper Motor Using At89c51 Using Proteus

Objective:

- To interface a stepper motor with the AT89C51 microcontroller using Proteus.

- To develop a program to rotate the stepper motor in a specified sequence.

- To understand the stepping sequence and control of stepper motors using a microcontroller.

Program:
ORG 0000H

UP: MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP UP

DELAY:MOV R4,#18

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

Experiment 4

AIM: 7 Segment Display Using At89c51 Using Proteus

Objective:

- To interface a 7-segment display with the AT89C51 microcontroller in Proteus.

- To write a program to display numeric digits on the 7-segment display.

- To understand the working of LED display interfacing and digital output control using the AT89C51.

Program:
ORG 000H

UP:MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0B0H

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2, #80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

DELAY: MOV R5,#10

H1:MOV R4,#180

H2:MOV R3,#255

H3:DJNZ R3,H3

DJNZ R4,H2

DJNZ R5,H1

RET

END

OUTPUT: